DYN4 SERVO JP5 i/o to 7i77 Board Pin-out (X-Axis)

| Symbol | Servo Drive Description ( JP5 header ) | Servo Drive I/ O | Drive JP5 Pin No. | 7i77 Pin No. | 7i77 TB No. | 7i77 Description | 7i77 I/ O |
| --- | --- | --- | --- | --- | --- | --- | --- |
| GND | Internal | NA | 1 |  |  | Ground |  |
| ABS_H | Absolute encoder auto home ( A circuit) | Input | 2 |  |  | Would like to use if possible. |  |
| ENA | Servo disable input ( A circuit) | Input | 3 | ? | TB5 | ENA+? | Output |
| +14VIN | External +14VDC input ( A circuit) | Input | 4 | N/A |  | Not Used |  |
| COM | Output photocoupler open collector ( B circuit) | Output | 5 |  |  |  |  |
| ONPOS | Servo on position output (B circuit) | Output | 6 |  |  | Use any 7i77 input |  |
| SRDY | Servo ready output (B circuit) | Output | 7 | ? | ? | Use any 7i77 input? |  |
| MO_POS | Position error monitor (E circuit) | Output | 8 |  |  | Would like to use if possible. |  |
| GND | Internal | NA | 9 |  |  | Ground |  |
| DIR - | Dir- , B- , CCW- pulse reference ( C circuit) | Input | 10 | N/A |  | Not Used |  |
| PUL+ | PUL+ , A+ , CW+ pulse reference ( C circuit) | Input | 11 | N/A |  | Not Used |  |
| GND | Internal | NA | 12 |  |  | Ground |  |
| AIN+ | Analog reference input positive ( D circuit) | Input | 13 | 4 | TB5 | Analog reference positive output | Output |
| DIN4 | Optional digital input 4 ( A circuit) | Input | 14 | N/A |  | Not Used |  |
| SHOLD | Servo hold input ( A circuit) | Input | 15 |  |  |  |  |
| +24VIN | External +24VDC input ( A circuit) | Input | 16 | PSU | PSU | From 24VDC PSU | PSU |
| +14VOUT | Servo internal +14VDC output | Output | 17 | N/A |  | Not Used |  |
| ALM | Servo alarm output ( B circuit) | Output | 18 | ? | ? | Use any 7i77 input |  |
| BKO | Holding brake output | Output | 19 |  |  | Z-Axis only |  |
| MO_ABS | Absolute encoder position monitor output ( E circuit) | Output | 20 |  |  | Would like to use if possible. |  |
| MO_IC | Servo drive current monitor output ( E circuit) | Output | 21 |  |  | Would like to use if possible. |  |
| DIR+ | DIR+ , B+ , CCW+ pulse reference ( C circuit) | Input | 22 | N/A |  | Not Used |  |
| PUL- | PUL- , A- ,CW- pulse reference ( C circuit) | Input | 23 | N/A |  | Not Used |  |
| +5VOUT | Servo drive internal +5VDC output | Output | 24 | N/A |  | Not Used |  |
| AIN- | Analog reference input negative ( D circuit) | Input | 25 | 3 | TB5 | Analog reference GND output | Output |