[MAIN PART OF THE CFF]

[MACHINE PROPERTIES]

name=Mach 3 with Touch-n-Go

model=No Subroutines

subroutines=0

comment_indicator=//

Machine Type=0

// Machine Types: 0 = plasma, 1 = oxyfuel, 2 = waterjet, 3 = laser, 4 = router

[PROCESS NAMES]

CUT1=Cut

SCRIBE1=Scribe

PUNCH1=Punch

[REVISION NUMBER]

$Revision: 1.1 $

[REVISION NOTES]

1.0 Original version from Generic 1.36

1.1 A bit of rephrasing and cleanups on the configuration comments

[MAIN]

//

////////////////////////////////////////////////////////////////////////////////

// The following variables can be initialized in this section for NestMaster.

// For TurboNest 2010 or ProNest 2010, they can be initialized here or set in

// a process parameter file. In addition, particularily for NestMaster, each

// of the parameters has a prompt line. If the // is removed from in front of

// the specific prompt lines, NestMaster (or TurboNest or ProNest) will prompt

// the user for that value when they output a cnc file.

////////////////////////////////////////////////////////////////////////////////

//

// The following section shows how the values are used in the Touch-n-Go sequence

//

// G28.1 Z($G28Z)

// G92 Z0

// G00 Z($SWITCHOFFSET)

// G92 Z0

// G00 Z($PIERCEHEIGHT)

// G04 P($PREHEAT) Only shows up if $PREHEAT>0

// M03

// G04 P($PIERCEDELAY) Only shows up if $PIERCEDELAY>0

// cutting occurs here

// M05

// G04 P($ENDDELAY) Only shows up if $ENDDELAY>0

//

IF $UNITS=INCH

// If not prompting, these are the default inch values

$G28Z := 3.0/25.4

$SWITCHOFFSET:=0.054

$PIERCEHEIGHT:=0.125

$SAFEZ:=2.0

ELSE

// If not prompting, these are the default metric values

$G28Z := 3.0

$SWITCHOFFSET:=2

$PIERCEHEIGHT:=4

$SAFEZ:=20.0

END

// These values are the same for inch or metric

$PREHEAT:=0.0

$PIERCEDELAY:=0.0

$ENDDELAY:=0.0

// If non-zero, SCODE will come out at the top of the file after the F1 line

$SCODE:=0

// $TOOL is used as the T value in the M06 line. If zero, the M06 line is omitted.

$TOOL:=0

// CONFIGTOUCHNGO can have 3 possible values:

// $CONFIGTOUCHNGO:=NONE Don't do Touch-N-Go

// $CONFIGTOUCHNGO:=PART Do a Touch-N-Go only on the first profile of a part

// $CONFIGTOUCHNGO:=ALL Do a Touch-N-Go for every profile start

$CONFIGTOUCHNGO:=NONE

//

// For the next several lines, if you want to prompt the user for the individual

// values, remove the // from in front of the line that you want prompting for.

// If you don't want to prompt the user for a value, then leave the // in front

// of the prompt for that value.

//

//$G28Z := 'NPROMPT(Enter G28.1 Z Value)

//$SWITCHOFFSET := 'NPROMPT(Enter Switch Offset Z Value)

//$PIERCEHEIGHT := 'NPROMPT(Enter Pierce Height)

//$SAFEZ := 'NPROMPT(Enter Safe Z Value)

//$PREHEAT := 'NPROMPT(Enter Preheat Time 0 for none)

//$PIERCEDELAY := 'NPROMPT(Enter Pierce Delay 0 for none)

//$ENDDELAY := 'NPROMPT(Enter End Delay 0 for none)

//$SCODE := 'NPROMPT(Enter S Code 0 for none)

//$TOOL := 'NPROMPT(Enter Tool number 0 for none)

// 0 => Use G41/G42/G40 on lines by themselves

// 1 => Use G41/G42 with leadin move, G40 with leadout

// 2 => Don't use G41/G42 (perhaps the CUTON in the machine triggers cutter comp?)

$CONFIGUSEKERFONMOVE:=0

// 0 => G40 on an arc move is acceptable

// 1 => Delay G40 to the next rapid if still in CW/CCW mode

$CONFIGNOG40ONARC:=0

// 0 => Use K value with kerf amount

// 1 => Use D value with G41/G42

// 2 => Don't include anything with G41/G42

// 3 => Use Kerf value as D value (typically used with TurboNest

$CONFIGUSEDVALUE:=0

// 0 => Use feedrate on a line by itself

// 1 => Use feedrate on the leadin motion line

// 2 => Use no feedrates

$CONFIGFEEDRATEONMOVE:=0

// 0 => Output Feedrate with each process start

// 1 => Suppress Feedrate if it hasn't changed

$CONFIGSUPRESSFEEDRATE:=0

// 0 => No G61/G64 codes

// 1 => Include G61/G64 exact stop codes based on angle change at end of motion

// 2 => Use G09 for exact stop check on each occurance

$CONFIGUSEEXACTSTOPCHECK:=0

// 0 => Start of Profile line numbers are on lines by themselves

// 1 => Start of Profile line numbers are on the process on rapid motion line

$CONFIGSOPLINENOLOCATION:=1

// 0 => Include G00/G01/G02/G03 on every motion line

// 1 => Suppress repeated G00/G01/G02/G03 codes

$CONFIGUSEMODALMOTIONCODES:=0

// 0 => Include X and Y on every motion line

// 1 => Suppress duplicate X, Y codes

// 2 => Include X and Y on all rapid motion, suppress on cutting motion

$CONFIGUSEMODALXY:=2

// CONFIGAHCISON can start out as 0, 1, 2, or 3

// 0 => implies that AHC defaults at start of program to off

// 1 => implies that AHC defaults at start of program to on

// 2 => implies that an AHC code should be used at the start in any case

// 3 => No AHC codes

// In most cases, 1 or 3 are the only usable choices.

$CONFIGAHCISON:=1

// 0 => I, J are absolute if X, Y are absolute, otherwise incremental

// 1 => I, J are always incremental

$CONFIGIJALWAYSINCREMENTAL:=1

// 0 => CUT OFF, CUT OFFUP are as normal

// 1 => At end of exterior profiles, skeleton cuts, and Crop cuts, always use OFFUP

$CONFIGFORCEEXTERIORTOOFFUP:=1

// Set a Zero point. By default, use $INITPTX, $INITPTY, which is where the

// starting point is located. ZEROPT_ allows setting a zero point separate

// from the initial point. To start the head out in the upper left of the

// sheet, with all dimensions in absolute from the lower left, set the init

// point in PN to the upper left, and use $ZEROPTX:=$SHEETMINX and

// $ZEROPTY:=$SHEETMINY

$ZEROPTX := $INITPTX

$ZEROPTY := $INITPTY

$AHCISON := $CONFIGAHCISON

{START OF FILE}

// Uncomment the next line to include a list of all parts, quantities on the list

//{OUTPUT PLP LIST INFO}

{PROGRAM INIT}

{VALIDATE PLATE}

{OUTPUT PROFILES}

{END OF FILE}

[SECTION PART OF THE CFF]

[START OF FILE]

[PLP INFO LINE]

// The next line might need comment characters adjusted if you're using the part list

{LINENO :M}(PART#:{ROOT PART NAME} QTY {PART QUANTITY ON NEST})

[ROOT PART NAME]

// ROOT PART NAME is the part file name without path (used only in PLP INFO LINE)

ARGUMENT=$ROOTPARTNAME

FORMAT=%s

[PART QUANTITY ON NEST]

// Number of times this part occurs on this nest (used only in PLP INFO LINE)

ARGUMENT=$PARTQUANTITYONNEST

FORMAT=%d

[PROGRAM INIT]

// Next two lines are example of how to prompt for a program number

//$PROGRAMNUMBER := 'NPROMPT(Enter Program Number)

//P{PROGRAM NUMBER}

{LINENO :M}(Filename: {ROOT FILE NAME})

{LINENO :M}(Date: {DATE})

{UNITS}

{MODE}

{LINENO :M}f#<_hal[plasmac.cut-feed-rate]>

{INITIALIZE CUT PARAMETERS}

IF $SCODE>0

{LINENO :M}{SCODE}

END

{RESET PARAMETERS}

[SCODE]

ARGUMENT=$SCODE

FORMAT=S%d

[UNITS]

// Common G codes for units are G70 or G20 for inch and G71 or G21 for metric

IF $UNITS=INCH

{LINENO :M}G20

ELSE

{LINENO :M}G21

END

[MODE]

IF $MODE=ABSOLUTE

{LINENO :M}G53 G90 G40

//{LINENO :M}G92{INITPTX}{INITPTY}

ELSE

{LINENO :M}G91 G40

END

[QUAD ARCS]

0

[ROOT FILE NAME]

ARGUMENT=$ROOTFILENAME

FORMAT=%s

[PROGRAM NUMBER]

ARGUMENT=$PROGRAMNUMBER

FORMAT=%d

[DATE]

ARGUMENT=$DATE

FORMAT=%s

[TIME]

ARGUMENT=$TIME

FORMAT=%s

[MATERIAL THICKNESS]

ARGUMENT=$MATERIALTHICKNESS * $CONVERSION

FORMAT={Z FORMAT}

[MATERIAL NAME]

ARGUMENT=$MATERIALNAME

FORMAT=%s

[PART CLASS]

ARGUMENT=$PART_CLASS

FORMAT=%s

[CNC NAME]

ARGUMENT=$FILENAME

FORMAT=%s

[NEST NUMBER]

ARGUMENT=$NESTNUMBER+1

FORMAT=%d

[NUMBER OF NESTS]

ARGUMENT=$NUMBER_OF_NESTS

FORMAT=%d

[TIMES TO RUN]

ARGUMENT=$CUTNUMBEROFTIMES

FORMAT=%d

[PRODUCTION TIME]

ARGUMENT=$PRODUCTIONTIME

FORMAT=%4.3f

[SHEET X]

ARGUMENT=$SHEETX * $CONVERSION

FORMAT={DIMENSION DISPLAY FORMAT}

[SHEET Y]

ARGUMENT=$SHEETY * $CONVERSION

FORMAT={DIMENSION DISPLAY FORMAT}

[STEP AND REPEAT START]

{LINENO :M}G97

[STEP AND REPEAT END]

{LINENO :M}G98

[VALIDATE PLATE]

IF $VALIDATEPLATE = TRUE

// Note that X and Y both need to be set each at each step for incremental

// output to work correctly.

$X := $INITPTX

$Y := $INITPTY

IF $INITPTLOCATION = LOWERLEFT

$X := $SHEETMAXX

$Y := $SHEETMAXY

ELSE

IF $INITPTLOCATION = LOWERRIGHT

$X := $SHEETMINX

$Y := $SHEETMAXY

ELSE

IF $INITPTLOCATION = UPPERRIGHT

$X := $SHEETMINX

$Y := $SHEETMINY

ELSE

$X := $SHEETMAXX

$Y := $SHEETMINY

END

END

END

{RAPID MOTION}

IF $INITPTLOCATION = LOWERLEFT

$X := $SHEETMINX

$Y := $SHEETMINY

ELSE

IF $INITPTLOCATION = LOWERRIGHT

$X := $SHEETMAXX

$Y := $SHEETMINY

ELSE

IF $INITPTLOCATION = UPPERRIGHT

$X := $SHEETMAXX

$Y := $SHEETMAXY

ELSE

$X := $SHEETMINX

$Y := $SHEETMAXY

END

END

END

{RAPID MOTION}

END

[END OF FILE]

{CANCEL CURRENT OFFSET}

{END OF PROGRAM}

[END OF PROGRAM]

IF $RETURNTOINIT=TRUE

{LINEAR MOTION TO INIT}

END

{LINENO :M}M30

[LINEAR MOTION TO INIT]

$X:=$INITPTX

$Y:=$INITPTY

{RAPID MOTION}

[OUTPUT START INTERIOR PROFILE]

// $PREKERFED variable tracks to see if a KERF LEFT or KERF RIGHT CALL is

// made so kerf off calls can be made only if needed

{CANCEL CURRENT OFFSET}

{SOP LINENO 0}

{OUTPUT TORCH SPACING}

{ACTIVATE NEXT OFFSET}

{PROCESS ON MOTION}

$DOTOUCHNGO:=0

IF $CONFIGTOUCHNGO=ALL

$DOTOUCHNGO:=1

END

{PART NAME COMMENT}

$PREKERFED:=1

$FORCEOFFUP:=0

[OUTPUT START EXTERIOR PROFILE]

{CANCEL CURRENT OFFSET}

{SOP LINENO 0}

{OUTPUT TORCH SPACING}

{ACTIVATE NEXT OFFSET}

{PROCESS ON MOTION}

$DOTOUCHNGO:=0

IF $CONFIGTOUCHNGO=ALL

$DOTOUCHNGO:=1

END

{PART NAME COMMENT}

$PREKERFED:=1

$FORCEOFFUP:=$CONFIGFORCEEXTERIORTOOFFUP

[OUTPUT START OPEN PROFILE]

{CANCEL CURRENT OFFSET}

{SOP LINENO 0}

{OUTPUT TORCH SPACING}

{ACTIVATE NEXT OFFSET}

{PROCESS ON MOTION}

$DOTOUCHNGO:=0

IF $CONFIGTOUCHNGO=ALL

$DOTOUCHNGO:=1

END

{PART NAME COMMENT}

$PREKERFED:=1

$FORCEOFFUP:=$CONFIGFORCEEXTERIORTOOFFUP

[OUTPUT START CROP]

{CANCEL CURRENT OFFSET}

{SOP LINENO 0}

{OUTPUT TORCH SPACING}

{ACTIVATE NEXT OFFSET}

{PROCESS ON MOTION}

$DOTOUCHNGO:=0

IF $CONFIGTOUCHNGO=ALL

$DOTOUCHNGO:=1

END

{PART NAME COMMENT}

$PREKERFED:=1

$FORCEOFFUP:=$CONFIGFORCEEXTERIORTOOFFUP

[OUTPUT TORCH SPACING]

IF {TORCH SPACING REQUIRED}

IF $NUMTORCHES = 1 || $NUMTORCHES = 0

(Set 1 torch.)

{OUTPUT STOP}

ELSE

IF $USEAUTOTORCHSPACING = TRUE

// Auto torch spacing commands go here.

END

(Set {NUMTORCHES} torches at {TORCHSPACING}{TORCHUNITS}.)

{OUTPUT STOP}

END

END

[OUTPUT END INTERIOR PROFILE]

IF {KERF STILL ON} && $CONFIGNOG40ONARC=0

{KERF OFF}

END

[OUTPUT END EXTERIOR PROFILE]

IF {KERF STILL ON} && $CONFIGNOG40ONARC=0

{KERF OFF}

END

[OUTPUT END OPEN PROFILE]

IF {KERF STILL ON} && $CONFIGNOG40ONARC=0

{KERF OFF}

END

[OUTPUT END CROP]

IF {KERF STILL ON} && $CONFIGNOG40ONARC=0

{KERF OFF}

END

[LINENO]

MODAL_ARGUMENT=$LINENUMBER

IF $USELINENUMBERS = EVERYLINE

{NWORD}

END

[SOP LINENO 0]

IF $USELINENUMBERS = STARTOFPROFILE && $CONFIGSOPLINENOLOCATION=0

{NWORD}

END

[SOP LINENO 1]

IF $USELINENUMBERS = STARTOFPROFILE && $CONFIGSOPLINENOLOCATION=1

{NWORD}

END

[PART NAME COMMENT]

$SAVEPARTNAME := $PARTNAME

$SAVENUM := $PARTSEQUENCENUMBER

IF $SAVENUM != $LASTSAVENUM || $SAVEPARTNAME != $LASTSAVEPARTNAME

IF $CONFIGTOUCHNGO=PART

$DOTOUCHNGO:=1

END

IF $PARTNAMEASCOMMENT = TRUE

IF $PARTSEQUENCENUMBER > 0

(Seq {PART SEQUENCE NUMBER} - {PARTNAME})

ELSE

({PARTNAME})

END

END

END

[PART SEQUENCE NUMBER]

ARGUMENT=$PARTSEQUENCENUMBER

FORMAT=%d

[PARTNAME]

ARGUMENT=$PARTNAME

FORMAT=%s

[CHANGE OFFSETS]

{CANCEL PREVIOUS OFFSET}

{ACTIVATE CURRENT OFFSET}

[CANCEL PREVIOUS OFFSET]

IF $PROCESS != $LASTPROCESS

IF $LASTPROCESS=SCRIBE1

//{LINENO :M}M70

END

END

[CANCEL CURRENT OFFSET]

IF $PROCESS != $NEXTPROCESS

IF $PROCESS=SCRIBE1

//{LINENO :M}M70

END

END

[ACTIVATE CURRENT OFFSET]

IF $PROCESS != $LASTPROCESS

IF ($PROCESS=PUNCH1 || $PROCESS=PUNCH2 || $PROCESS=PUNCH3 || $PROCESS=PUNCH4 || $PROCESS=PUNCH5 || $PROCESS=PUNCH6) && $STOPBEFOREPUNCH = TRUE

{OUTPUT STOP}

ELSE

IF ($PROCESS=SCRIBE1 || $PROCESS=SCRIBE2) && $STOPBEFORESCRIBE = TRUE

{OUTPUT STOP}

ELSE

IF ($PROCESS=CUT1 || $PROCESS=CUT2 || $PROCESS=CUT3) && $STOPBEFORECUT = TRUE

{OUTPUT STOP}

END

END

END

IF ($PROCESS=CUT1 || $PROCESS=PUNCH1) && $TOOL>0

{LINENO :M}M06 T{TOOL}

{SAFE Z}

END

END

[ACTIVATE NEXT OFFSET]

IF $PROCESS != $NEXTPROCESS

IF ($NEXTPROCESS=PUNCH1 || $NEXTPROCESS=PUNCH2 || $NEXTPROCESS=PUNCH3 || $NEXTPROCESS=PUNCH4 || $NEXTPROCESS=PUNCH5 || $NEXTPROCESS=PUNCH6) && $STOPBEFOREPUNCH = TRUE

{OUTPUT STOP}

ELSE

IF ($NEXTPROCESS=SCRIBE1 || $NEXTPROCESS=SCRIBE2) && $STOPBEFORESCRIBE = TRUE

{OUTPUT STOP}

ELSE

IF ($NEXTPROCESS=CUT1 || $NEXTPROCESS=CUT2 || $NEXTPROCESS=CUT3) && $STOPBEFORECUT = TRUE

{OUTPUT STOP}

END

END

END

IF ($NEXTPROCESS=CUT1 || $NEXTPROCESS=PUNCH1) && $TOOL>0

{LINENO :M}M06 T{TOOL}

{SAFE Z}

END

END

[PROCESS ON MOTION]

{SOP LINENO 1 :M}{RAPID MOTION}

[RAPID MOTION]

$MOTIONTYPE:=0

IF $CONFIGUSEMODALXY=2

{LINENO :M}{KERF MOVE :M}{RAPID MOTION CODE :M}{CURRENTX}{CURRENTY}

ELSE

{LINENO :M}{KERF MOVE :M}{RAPID MOTION CODE :M}{CURRENTX :M}{CURRENTY :M}

END

[PRE MOTION]

// Calls at the start of the MOTION lines

// This keeps the MOTION sections not quite as wide

{EXACT STOP :M}{KERF MOVE :M}

[POST MOTION]

// Calls at the end of the MOTION lines

{ADD FEEDRATE :M}

[LINEAR MOTION]

$MOTIONTYPE:=1

{FEEDRATE LINE}

IF $CONFIGUSEKERFONMOVE=0

{KERF DIRECTION :M}

ELSE

IF $INLEADOUT=TRUE && $NEXTPROFILEMOTION=NONE

$KERFDIR:=OFF

END

END

{LINENO :M}{PRE MOTION :M}{LINEAR MOTION CODE :M}{CURRENTX :M}{CURRENTY :M}{POST MOTION :M}

[CLOCKWISE MOTION]

$MOTIONTYPE:=2

{CHECK FOR ARC TOO SMALL}

IF $ARCTOOSMALL=1

{LINEAR MOTION}

ELSE

{FEEDRATE LINE}

IF $CONFIGUSEKERFONMOVE=0

{KERF DIRECTION :M}

ELSE

IF $INLEADOUT=TRUE && $NEXTPROFILEMOTION=NONE && $CONFIGNOG40ONARC=0

$KERFDIR:=OFF

END

END

{LINENO :M}{PRE MOTION :M}{CLOCKWISE MOTION CODE :M}{CURRENTX :M}{CURRENTY :M}{CURRENTI}{CURRENTJ}{POST MOTION :M}

END

[COUNTERCLOCKWISE MOTION]

$MOTIONTYPE:=3

{CHECK FOR ARC TOO SMALL}

IF $ARCTOOSMALL=1

{LINEAR MOTION}

ELSE

{FEEDRATE LINE}

IF $CONFIGUSEKERFONMOVE=0

{KERF DIRECTION :M}

ELSE

IF $INLEADOUT=TRUE && $NEXTPROFILEMOTION=NONE && $CONFIGNOG40ONARC=0

$KERFDIR:=OFF

END

END

{LINENO :M}{PRE MOTION :M}{COUNTERCLOCKWISE MOTION CODE :M}{CURRENTX :M}{CURRENTY :M}{CURRENTI}{CURRENTJ}{POST MOTION :M}

END

[EXACT STOP]

// This section decides if the machine should maintain feedrate at the end of the motion

// or if it should come to a stop. The degree of angle change determines this

IF $CONFIGUSEEXACTSTOPCHECK=0

$EXACTSTOPCHECK:=2

END

IF $NEXTPROFILEMOTION=NONE

$ANGLECHANGE:=0

ELSE

$ANGLECHANGE:='ABS($ENDTANGENTANGLE-$NEXTSTARTTANGENTANGLE)

END

IF $ANGLECHANGE>180

$ANGLECHANGE:='ABS($ANGLECHANGE-360.0)

END

IF $CONFIGUSEEXACTSTOPCHECK=2

$EXACTSTOPCHECK:=0

END

// The number in the next line determines when an EXACT STOP needs to be inserted

// and when it should maintain feedrate

IF $ANGLECHANGE>=$EXACTSTOPANGLE

// Use exact stop check mode

IF $EXACTSTOPCHECK=0

IF $CONFIGUSEEXACTSTOPCHECK=2

// One-shot exact stop

G09

ELSE

// Modal exact stop

G64

END

$EXACTSTOPCHECK:=1

END

ELSE

// Continuous (cut) mode

IF $EXACTSTOPCHECK=1

G61

$EXACTSTOPCHECK:=0

END

END

[RAPID MOTION CODE]

IF ($CONFIGUSEMODALMOTIONCODES=0) || ($CURRENTMOTION!=0)

G00

$CURRENTMOTION:=0

END

[LINEAR MOTION CODE]

IF ($CONFIGUSEMODALMOTIONCODES=0) || ($CURRENTMOTION!=1)

G01

$CURRENTMOTION:=1

END

[CLOCKWISE MOTION CODE]

IF ($CONFIGUSEMODALMOTIONCODES=0) || ($CURRENTMOTION!=2)

IF {NORMAL AXIS DIRECTION}

G02

ELSE

G03

END

$CURRENTMOTION:=2

END

[COUNTERCLOCKWISE MOTION CODE]

IF ($CONFIGUSEMODALMOTIONCODES=0) || ($CURRENTMOTION!=3)

IF {NORMAL AXIS DIRECTION}

G03

ELSE

G02

END

$CURRENTMOTION:=3

END

[INIT PT MOTION CODE]

{RAPID MOTION CODE}

[ADD FEEDRATE]

IF $FEEDRATE > 0.0 && $FEEDRATE != $LASTFEEDRATE && $CONFIGFEEDRATEONMOVE=1

{FEEDRATE :M}

$FEEDRATE:=$FEEDRATE

END

[FEEDRATE LINE]

IF $FEEDRATE > 0.0 && $FEEDRATE != $LASTFEEDRATE && $CONFIGFEEDRATEONMOVE=0

{LINENO :M}{FEEDRATE :M}

$FEEDRATE:=$FEEDRATE

END

[ADD KERF VALUE]

IF $KERF > 0.0 && $KERF != $LASTKERF

{KERF VALUE}

$KERF:=$KERF

END

[OUTPUT KERF LEFT]

// Any call from Pronest to this or the next section says that this part was not

// prekerfed. If not prekerfed, then we need to cancel the Kerf at the end of

// the cut.

$PREKERFED:=0

[OUTPUT KERF RIGHT]

$PREKERFED:=0

[KERF LEFT]

IF $CONFIGUSEKERFONMOVE<2

IF ($KERF <= 0 && $CONFIGUSEDVALUE=0) || $CONFIGUSEDVALUE=2

IF $CONFIGUSEKERFONMOVE=0

{LINENO :M}G41

ELSE

G41

END

ELSE

IF $CONFIGUSEKERFONMOVE=0

{LINENO :M}G41{KERF VALUE :M}

ELSE

G41{KERF VALUE :M}

END

$KERF:=$KERF

END

END

$KERFSTATUS:=41

[KERF RIGHT]

IF $CONFIGUSEKERFONMOVE<2

IF ($KERF <= 0 && $CONFIGUSEDVALUE=0) || $CONFIGUSEDVALUE=2

IF $CONFIGUSEKERFONMOVE=0

{LINENO :M}G42

ELSE

G42

END

ELSE

IF $CONFIGUSEKERFONMOVE=0

{LINENO :M}G42{KERF VALUE :M}

ELSE

G42{KERF VALUE :M}

$KERF:=$KERF

END

END

$KERFSTATUS:=42

[KERF OFF]

IF $CONFIGUSEKERFONMOVE<2

IF $KERFSTATUS!=40

IF $CONFIGUSEKERFONMOVE=0

{LINENO :M}G40

ELSE

G40

END

END

END

$KERFSTATUS:=40

[KERF MOVE]

{KERF DIRECTION}

[KERF DIRECTION]

IF ($PROCESS != SCRIBE1) && ($PROCESS != SCRIBE2) && ($PREKERFED=0)

IF $KERFDIR = LEFT && $KERFSTATUS!=41

{KERF LEFT}

END

IF $KERFDIR = RIGHT && $KERFSTATUS!=42

{KERF RIGHT}

END

IF $KERFDIR = OFF && $KERFSTATUS!=40

{KERF OFF}

END

END

[OUTPUT STOP]

{LINENO :M}M00

[OUTPUT AHCON]

{AHCON}

[OUTPUT AHCOFF]

{AHCOFF}

[AHCON CODE]

{LINENO :M}M51

[AHCOFF CODE]

{LINENO :M}

[AHCON]

IF ($AHCISON = 0 || $AHCISON = 2) && $DISABLEAHC=0

{AHCON CODE}

$AHCISON:=1

END

[FORCE AHCON]

IF ($AHCISON = 0 || $AHCISON = 2)

{AHCON CODE}

$AHCISON:=1

END

[AHCOFF]

IF ($AHCISON = 1 || $AHCISON = 2)

{AHCOFF CODE}

$AHCISON:=0

END

[CHECK AHC]

// If Pronest thinks it's supposed to be on ($AHC=ON), and it's not, and we're

// not disabling it through the Process Parameter file, turn it on

IF $AHC=ON && $AHCISON=0 && $DISABLEAHC=0

{AHCON}

END

// If Pronest thinks it's supposed to be off ($AHC=OFF), and it's on, turn it off

IF $AHC=OFF && $AHCISON!=0 || $DISABLEAHC=1

{AHCOFF}

END

[START LEADIN]

// This section is here as a place holder and so the name is already in place

[END LEADIN]

// This section is here as a place holder and so the name is already in place

[START LEADOUT]

// This section is here as a place holder and so the name is already in place

[END LEADOUT]

// This section is here as a place holder and so the name is already in place

[START TAB]

$INTAB := 1

[END TAB]

$INTAB := 0

{END LEADIN}

[CUT SUB]

// This section may be useful if an integer value is used to initiate cutting.

// This is often an M98P___ call to a subroutine already stored in the machine

ARGUMENT=$CUTSUB

FORMAT=M98P%d

[CUT CONDITION]

// This section may be useful if a cutting condition needs to be applied

// separately from the CUT SUB value.

ARGUMENT=$CUTCONDITION

FORMAT=E%d

[TOUCH AND GO]

IF $DOTOUCHNGO=1

{PIERCE HEIGHT}

{G28 Z}

{LINENO :M}G92 Z0.0

{SWITCH OFFSET}

{LINENO :M}G92 Z0.0

END

{PIERCE HEIGHT}

[PIERCE HEIGHT]

ARGUMENT=$PIERCEHEIGHT

FORMAT={LINENO :M}{RAPID MOTION CODE :M} Z{Z FORMAT}

[G28 Z]

ARGUMENT=$G28Z

FORMAT={LINENO :M}G28.1 Z{Z FORMAT}

[SWITCH OFFSET]

ARGUMENT=$SWITCHOFFSET

FORMAT={LINENO :M}{RAPID MOTION CODE :M} Z{Z FORMAT}

[OUTPUT CUT1 ON]

// CUT1 ON codes go in this section.

{FEEDRATE LINE}

{CHECK AHC}

IF $CONFIGUSEKERFONMOVE=0

{KERF DIRECTION :M}

END

{TOUCH AND GO}

{OUTPUT PREHEAT}

{LINENO :M}m3 $0 s1

{OUTPUT PIERCEDELAY}

[OUTPUT CUT1 OFF]

IF $FORCEOFFUP=0

// Turn off the cutting device, but leave the head at cutting height

{CHECK AHC}

{LINENO :M}M05

{OUTPUT ENDDELAY}

// Should we really call Safe Z here?

{SAFE Z}

IF $PREKERFED=0 && $INTAB=0 && ($CONFIGNOG40ONARC=0 || $CURRENTMOTION<2)

{KERF OFF}

END

$KERF:=0.0

{FORCE AHCON}

IF $CONFIGSUPRESSFEEDRATE=0

$FEEDRATE:=0.0

END

ELSE

{OUTPUT CUT1 OFFUP}

END

[OUTPUT CUT1 OFFUP]

// Turn off the cutting device, and raise the head to travel height

{CHECK AHC}

{LINENO :M}M05

{OUTPUT ENDDELAY}

{SAFE Z}

IF $PREKERFED=0 && $INTAB=0 && ($CONFIGNOG40ONARC=0 || $CURRENTMOTION<2)

{KERF OFF}

END

$KERF:=0.0

{FORCE AHCON}

IF $CONFIGSUPRESSFEEDRATE=0

$FEEDRATE:=0.0

END

[SAFE Z]

ARGUMENT=$SAFEZ

FORMAT={LINENO :M}{RAPID MOTION CODE :M} Z{Z FORMAT}

[OUTPUT SCRIBE1 ON]

// Scribing or Marking commands go in this section

$PREKERFED:=1

{CANCEL PREVIOUS OFFSET}

{SOP LINENO 0}

IF $PARTNAMEASCOMMENT = TRUE

{PART NAME COMMENT}

END

{ACTIVATE CURRENT OFFSET}

{PROCESS ON MOTION}

{LINENO :M}M08

[OUTPUT SCRIBE1 OFF]

{LINENO :M}M07

$KERF:=0.0

IF $CONFIGSUPRESSFEEDRATE=0

$FEEDRATE:=0.0

END

[OUTPUT PUNCH1 ON]

// Depending on customer choice, this could be a piercing routine or a point

// marking routine. Or duplicate the section and make both.

$PREKERFED:=1

{CANCEL PREVIOUS OFFSET}

{SOP LINENO 0}

IF $PARTNAMEASCOMMENT = TRUE

{PART NAME COMMENT}

END

{ACTIVATE CURRENT OFFSET}

{PROCESS ON MOTION}

{TOUCH AND GO}

{OUTPUT PREHEAT}

{LINENO :M}M03

{OUTPUT PIERCEDELAY}

{LINENO :M}M05

{OUTPUT ENDDELAY}

{SAFE Z}

$KERF:=0.0

IF $CONFIGSUPRESSFEEDRATE=0

$FEEDRATE:=0.0

END

[COMMENT FORMAT]

(%s)

[FORMAT PART OF THE CFF]

[INITPTX]

ARGUMENT=($INITPTX - $ZEROPTX) * $CONVERSION

FORMAT=X{XY FORMAT}

[INITPTY]

ARGUMENT=($INITPTY - $ZEROPTY) * $CONVERSION

FORMAT=Y{XY FORMAT}

[CONVERSION]

IF $UNITS=INCH

1.00

ELSE

25.40

END

[NWORD]

// Use N%d for no leading zeros

// Use N%03d for 3 digit N numbers

// Use N%04d for 4 digit N numbers

FORMAT=N%d{SP}

ARGUMENT=$LINENUMBER

[NUMTORCHES]

ARGUMENT=$NUMTORCHES

FORMAT=%d

[TORCHSPACING]

ARGUMENT=$TORCHSPACING * $CONVERSION

FORMAT=%1.3lf

[TORCHUNITS]

IF $UNITS = INCH

in

ELSE

mm

END

[XY FORMAT]

IF $UNITS=INCH

IF $PRECISION=NORMAL

%1.3lf

ELSE

%1.4lf

END

ELSE

IF $PRECISION=NORMAL

%1.2lf

ELSE

%1.3lf

END

END

[IJ FORMAT]

IF $UNITS=INCH

IF $PRECISION=NORMAL

%1.3lf

ELSE

%1.4lf

END

ELSE

IF $PRECISION=NORMAL

%1.2lf

ELSE

%1.3lf

END

END

[Z FORMAT]

IF $UNITS=INCH

IF $PRECISION=NORMAL

%1.3lf

ELSE

%1.4lf

END

ELSE

IF $PRECISION=NORMAL

%1.2lf

ELSE

%1.3lf

END

END

[DIMENSION DISPLAY FORMAT]

IF $UNITS=INCH

IF $PRECISION=NORMAL

%1.3lf

ELSE

%1.4lf

END

ELSE

IF $PRECISION=NORMAL

%1.2lf

ELSE

%1.3lf

END

END

[FEEDRATE FORMAT]

IF $UNITS=INCH

%1.1lf

ELSE

%1.0lf

END

[DWELL FORMAT]

%1.2lf

[XYIJ DECIMAL SHIFT]

1

[FEEDRATE DECIMAL SHIFT]

1

[KERF DECIMAL SHIFT]

1

[DWELL DECIMAL SHIFT]

1

[ANGLE DECIMAL SHIFT]

1

[TOOL]

ARGUMENT=$TOOL

FORMAT=%d

[CURRENTX]

IF $CONFIGUSEMODALXY>0

MODAL_ARGUMENT=$X

END

IF $MODE=ABSOLUTE

ARGUMENT=($X - $ZEROPTX) * $CONVERSION * $XYIJDECIMALSHIFT

ELSE

ARGUMENT=($X - $LASTX) * $CONVERSION * $XYIJDECIMALSHIFT

END

FORMAT=X{XY FORMAT}

[CURRENTY]

IF $CONFIGUSEMODALXY>0

MODAL_ARGUMENT=$Y

END

IF $MODE=ABSOLUTE

ARGUMENT=($Y - $ZEROPTY) * $CONVERSION * $XYIJDECIMALSHIFT

ELSE

ARGUMENT=($Y - $LASTY) * $CONVERSION * $XYIJDECIMALSHIFT

END

FORMAT=Y{XY FORMAT}

[CURRENTI]

IF $MODE=ABSOLUTE && $CONFIGIJALWAYSINCREMENTAL=0

ARGUMENT=($I - $ZEROPTX) * $CONVERSION * $XYIJDECIMALSHIFT

ELSE

ARGUMENT=($I - $LASTX) * $CONVERSION * $XYIJDECIMALSHIFT

END

FORMAT=I{IJ FORMAT}

[CURRENTJ]

IF $MODE=ABSOLUTE && $CONFIGIJALWAYSINCREMENTAL=0

ARGUMENT=($J - $ZEROPTY) * $CONVERSION * $XYIJDECIMALSHIFT

ELSE

ARGUMENT=($J - $LASTY) * $CONVERSION * $XYIJDECIMALSHIFT

END

FORMAT=J{IJ FORMAT}

[FEEDRATE]

MODAL_ARGUMENT=$FEEDRATE

ARGUMENT=$FEEDRATE * $CONVERSION * $FEEDRATEDECIMALSHIFT

FORMAT=F{FEEDRATE FORMAT}

[KERF VALUE]

IF $CONFIGUSEDVALUE=1

// D value as kerf

ARGUMENT=$DVALUE

FORMAT=D%02d

ELSE

IF $CONFIGUSEDVALUE=0

// Kerf as actual kerf (typically with a leading K)

MODAL_ARGUMENT=$KERF

ARGUMENT=$KERF * $CONVERSION * $KERFDECIMALSHIFT

FORMAT=K{XY FORMAT}

ELSE

IF $CONFIGUSEDVALUE=3

// For TurboNest CFFs that want a D register but can't use a Process

// Parameter column, use the Kerf column as the D register number

ARGUMENT=$KERF * $CONVERSION

FORMAT=D%02.0lf

END

END

END

[OUTPUT DWELL]

ARGUMENT=$DWELL * $DWELLDECIMALSHIFT

// Dwell times are usually specified with an F, X, or P value

FORMAT={LINENO :M}G04F{DWELL FORMAT}

[OUTPUT PREHEAT]

IF $PREHEAT>0

ARGUMENT=$PREHEAT * $DWELLDECIMALSHIFT

FORMAT={LINENO :M}G04 P{DWELL FORMAT}

END

[OUTPUT PIERCEDELAY]

IF $PIERCEDELAY>0

ARGUMENT=$PIERCEDELAY * $DWELLDECIMALSHIFT

FORMAT={LINENO :M}G04 P{DWELL FORMAT}

END

[OUTPUT ENDDELAY]

IF $ENDDELAY>0

ARGUMENT=$ENDDELAY * $DWELLDECIMALSHIFT

FORMAT={LINENO :M}G04 P{DWELL FORMAT}

END

[CONDITIONS PART OF THE CFF]

[MAXLINENO]

9999

[TORCH SPACING REQUIRED]

($TORCHSPACING != $PREVTORCHSPACING || $NUMTORCHES != $PREVNUMTORCHES)

[KERF STILL ON]

$KERFDIR = LEFT || $KERFDIR = RIGHT

[MOTION REQUIRED]

(($X - $LASTX)^2 + ($Y - $LASTY)^2)^0.5 > 0.00001

[NORMAL AXIS DIRECTION]

(($AXIS_SWAP_TYPE=NONE) || ($AXIS_SWAP_TYPE=SWAP90) || ($AXIS_SWAP_TYPE=SWAP180) || ($AXIS_SWAP_TYPE=SWAP270))

[INSIDE CW ARC TOO SMALL FOR KERF]

($CONFIGUSEDVALUE<3 && ($MOTIONTYPE=2 && $KERFDIR=RIGHT && $RADIUS<$KERF*0.5))

[INSIDE CCW ARC TOO SMALL FOR KERF]

($CONFIGUSEDVALUE<3 && ($MOTIONTYPE=3 && $KERFDIR=LEFT && $RADIUS<$KERF*0.5))

[ARC TOO LINEAR]

({CHORD LENGTH}<(($RADIUS^2-($RADIUS-0.002)^2)^0.5)*2 && $RADIUS>3 && $ARCANGLE<10)

[ARC RADIUS TOO SMALL]

($RADIUS < 0.008000 || ({CHORD LENGTH} < 0.00200 && $ARCANGLE<10))

[CHECK FOR ARC TOO SMALL]

$ARCTOOSMALL:=0

IF {INSIDE CW ARC TOO SMALL FOR KERF}

$ARCTOOSMALL:=1

END

IF {INSIDE CCW ARC TOO SMALL FOR KERF}

$ARCTOOSMALL:=1

END

IF {ARC TOO LINEAR}

$ARCTOOSMALL:=1

END

IF {ARC RADIUS TOO SMALL}

$ARCTOOSMALL:=1

END

[RAPID INIT MOTION ALLOWED]

(($X - $INITPTX)^2 + ($Y - $INITPTY)^2)^0.5 > 0.0100 && $PROCESSONSTATE = FALSE

[ARC RADIUS]

(($I - $LASTX)^2 + ($J - $LASTY)^2)^0.5

[CHORD LENGTH]

(($X - $LASTX)^2 + ($Y - $LASTY)^2)^0.5

[CNC TOKEN DEBUG]

// See FogBugz #2923 for details

0

[SP]

ARGUMENT=$SPACE

FORMAT=%c

[CHARACTER REPLACE TABLE]

(|_

)|_

[CFFDIR]

IF $CFFDIR=none

$CFFDIR:='SUBSTRING($CFFNAME,'RANGE(0,'TEXTLEN($CFFNAME)-'TEXTLEN($ROOTCFFNAME)-6))

END

ARGUMENT=$CFFDIR

FORMAT=%s

// As of the 8.3 release version, the CFFDIR variable is optional in the user

// defined variables. In all prior release versions, it is required.

[ROOT CFF NAME]

ARGUMENT=$ROOTCFFNAME

FORMAT=%s

[POST PROCESSING]

// Uncomment if an executable is needed

//'EXECUTE "{CFFDIR}\{ROOT CFF NAME}.exe" "{CNC NAME}"

[USER DEFINED VARIABLES]

SAVEPARTNAME|STRING|0

SAVENUM|INTEGER|0

DX|DOUBLE|0.0

DY|DOUBLE|0.0

EXACTSTOPCHECK|INTEGER|0

ANGLECHANGE|DOUBLE|0.0

KERFSTATUS|INTEGER|0

PREKERFED|INTEGER|0

AHCISON|INTEGER|3

AHC|STRING|ON

SPACE|INTEGER|32

CONFIGTOUCHNGO|STRING|ALL

CONFIGUSEDVALUE|INTEGER|0

CONFIGUSEKERFONMOVE|INTEGER|0

CONFIGNOG40ONARC|INTEGER|0

CONFIGFEEDRATEONMOVE|INTEGER|0

CONFIGSUPRESSFEEDRATE|INTEGER|0

CONFIGUSEEXACTSTOPCHECK|INTEGER|0

CONFIGUSEMODALMOTIONCODES|INTEGER|0

CONFIGUSEMODALXY|INTEGER|0

CONFIGAHCISON|INTEGER|0

CONFIGIJALWAYSINCREMENTAL|INTEGER|1

CONFIGSOPLINENOLOCATION|INTEGER|0

CONFIGFORCEEXTERIORTOOFFUP|INTEGER|1

FORCEOFFUP|INTEGER|0

ZEROPTX|DOUBLE|0.0

ZEROPTY|DOUBLE|0.0

CFFDIR|STRING|none

CURRENTMOTION|INTEGER|-1

MOTIONTYPE|INTEGER|-1

INTAB|INTEGER|0

ARCTOOSMALL|INTEGER|0

DOTOUCHNGO|INTEGER|1

PossibleProcParamsStartHere|INTEGER|0

CUTSUB|INTEGER|0

CUTCONDITION|INTEGER|0

ProcParamColsStartHere|INTEGER|0

EXACTSTOPANGLE|DOUBLE|45.0

FEEDRATE|DOUBLE|0.0

DVALUE|INTEGER|0

DISABLEAHC|INTEGER|0

KERF|DOUBLE|0.0

DWELL|DOUBLE|0.0

PIERCEHEIGHT|DOUBLE|0.0

SWITCHOFFSET|DOUBLE|0.054

G28Z|DOUBLE|0.0

SAFEZ|DOUBLE|2.0

PREHEAT|DOUBLE|0.0

PIERCEDELAY|DOUBLE|0.4

ENDDELAY|DOUBLE|0.0

TOOL|INTEGER|7

SCODE|INTEGER|500