# Generated by PNCconf at Thu May 16 21:32:27 2019

# If you make changes to this file, they will be

# overwritten when you run PNCconf again

[EMC]

MACHINE = EMCOTURN220

DEBUG = 0

[DISPLAY]

DISPLAY = gmoccapy

EMBED_TAB_NAME = GladeVCP

EMBED_TAB_COMMAND = halcmd loadusr -Wn gladevcp gladevcp -c gladevcp -H gvcp_call_list.hal -x {XID} gvcp-panel.ui

EMBED_TAB_NAME = ATC

EMBED_TAB-LOCATION = BOX_CUSTOM_4

POSITION_OFFSET = RELATIVE

POSITION_FEEDBACK = ACTUAL

MAX_FEED_OVERRIDE = 1.500000

MAX_SPINDLE_OVERRIDE = 1.500000

MIN_SPINDLE_OVERRIDE = 0.500000

INTRO_GRAPHIC = linuxcnc.gif

INTRO_TIME = 5

PROGRAM_PREFIX = /home/emco/linuxcnc/nc_files

PYVCP = pyvcp-panel.xml

INCREMENTS = 5mm 1mm .5mm .1mm .05mm .01mm .005mm

LATHE = 1

POSITION_FEEDBACK = ACTUAL

DEFAULT_LINEAR_VELOCITY = 6.000000

MAX_LINEAR_VELOCITY = 25.000000

MIN_LINEAR_VELOCITY = 0.500000

DEFAULT_ANGULAR_VELOCITY = 12.000000

MAX_ANGULAR_VELOCITY = 180.000000

MIN_ANGULAR_VELOCITY = 1.666667

EDITOR = gedit

GEOMETRY = xyz

BACK_TOOL_LATHE = 1

[FILTER]

PROGRAM_EXTENSION = .png,.gif,.jpg Greyscale Depth Image

PROGRAM_EXTENSION = .py Python Script

png = image-to-gcode

gif = image-to-gcode

jpg = image-to-gcode

py = python

[TASK]

TASK = milltask

CYCLE_TIME = 0.010

[RS274NGC]

PARAMETER_FILE = linuxcnc.var

USER_M_PATH = /home/emco/linuxcnc/configs/EMCOTURN220/M-codes/

[EMCMOT]

EMCMOT = motmod

COMM_TIMEOUT = 1.0

COMM_WAIT = 0.010

SERVO_PERIOD = 1000000

[HOSTMOT2]

# **** This is for info only ****

# DRIVER0=hm2_eth

# BOARD0=7i76e

[HAL]

HALUI = halui

HALFILE = EMCOTURN220.hal

HALFILE = custom.hal

POSTGUI_HALFILE = postgui_call_list.hal

SHUTDOWN = shutdown.hal

[HALUI]

[TRAJ]

AXES = 3

COORDINATES = X Z

LINEAR_UNITS = mm

ANGULAR_UNITS = degree

CYCLE_TIME = 0.010

DEFAULT_VELOCITY = 11.67

MAX_VELOCITY = 116.67

[EMCIO]

EMCIO = io

CYCLE_TIME = 0.100

TOOL_TABLE = tool.tbl

#********************

# Axis X

#********************

[AXIS_0]

TYPE = LINEAR

HOME = 0.0

FERROR = 10.0

MIN_FERROR = 1.0

MAX_VELOCITY = 116.666666667

MAX_ACCELERATION = 750.0

# The values below should be 25% larger than MAX_VELOCITY and MAX_ACCELERATION

# If using BACKLASH compensation STEPGEN_MAXACCEL should be 100% larger.

STEPGEN_MAXVEL = 145.83

STEPGEN_MAXACCEL = 937.50

P = 1000.0

I = 0.0

D = 0.0

FF0 = 0.0

FF1 = 1.0

FF2 = 0.0

BIAS = 0.0

DEADBAND = 0.0

MAX_OUTPUT = 0.0

# these are in nanoseconds

DIRSETUP   = 1000

DIRHOLD    = 1000

STEPLEN    = 1000

STEPSPACE  = 1000

STEP_SCALE = -2000.0

BACKLASH = 0.015

MIN_LIMIT = -56.0

MAX_LIMIT = 0.0

HOME_OFFSET = 0.000000

HOME_SEARCH_VEL = -35.000000

HOME_LATCH_VEL = -1.500000

HOME_FINAL_VEL = 0.500000

HOME_USE_INDEX = NO

HOME_SEQUENCE = 1

#********************

# Axis Z

#********************

[AXIS_2]

TYPE = LINEAR

HOME = 0.0

FERROR = 10.0

MIN_FERROR = 1.0

MAX_VELOCITY = 116.666666667

MAX_ACCELERATION = 750.0

# The values below should be 25% larger than MAX_VELOCITY and MAX_ACCELERATION

# If using BACKLASH compensation STEPGEN_MAXACCEL should be 100% larger.

STEPGEN_MAXVEL = 145.83

STEPGEN_MAXACCEL = 937.50

P = 1000.0

I = 0.0

D = 0.0

FF0 = 0.0

FF1 = 1.0

FF2 = 0.0

BIAS = 0.0

DEADBAND = 0.0

MAX_OUTPUT = 0.0

# these are in nanoseconds

DIRSETUP   = 1000

DIRHOLD    = 1000

STEPLEN    = 1000

STEPSPACE  = 1000

STEP_SCALE = 2000.0

BACKLASH = 0.01

MIN_LIMIT = -175.0

MAX_LIMIT = 0.01

HOME_OFFSET = 0.000000

HOME_SEARCH_VEL = -35.000000

HOME_LATCH_VEL = -1.500000

HOME_FINAL_VEL = 0.500000

HOME_USE_INDEX = NO

HOME_SEQUENCE = 0

#********************

# Spindle

#********************

[SPINDLE_9]

P = 10.50

I = 0.1

D = 0.15

FF0 = 1.07

FF1 = 0.0

FF2 = 0.0

BIAS = 0.0

DEADBAND = 0.0

MAX_OUTPUT = 6000

ENCODER_SCALE = 4096

OUTPUT_SCALE = 5700

OUTPUT_MIN_LIMIT = 0.0

OUTPUT_MAX_LIMIT = 6000