LinuxCNC Ver. 2.7.14

Stepconf Wizard settings as follows.

Base Information

Machine Name:	Grand-Pa_Prb_Estp_V1.3

Configuration Dir:	~/linuxcnc/configs/Grand-Pa_Prb_Estp_V1.3

Axis Config:		XYZ

Machine Units:	Inch

Driver Type =		Other

Step Time = 		6000 ns

Step Space =		6000 ns

Direct. Hold =		5000 ns

Direct. Setup =	5000 ns

One Parport:		*

Base per Max Jit:	15000 ns

Min Base Period:	21000 ns

Max step rate:		23809 Hz

Parallel Port 1

Outputs		Inv.		Inputs			Inv.

Pin 1 = ESTOP Out			Pin 10 = Unused

Pin 2 = x step				Pin 11 = Unused

Pin 3 = x dir				Pin 12 = Unused

Pin 4 = y step				Pin 13 = ESTOP in

Pin 5 = y dir				Pin 15 = Probe in

Pin 6 = z step

Pin 7 = z dir

Pin 8 = Unused			Parport Base Address = 0

Pin 9 = Unused			Output pinout presets = Sherline

Pin 14 = Spindle CW   X

Pin 16 = Unused

Pin 17 = Unused

Options

Include Halui = checked		include custom PyVCP = checked

Onscreen prompt for manual tool change = checked

Axis X, Y & Z

Motor steps per revolution:		200

Driver Microstepping:			16 (X, Y & Z set to same)

Pulley teeth:				1: 1 (X, Y & Z set to same) (Direct Drive)

Leadscrew Pitch:			20 rev/in

Maximum Velocity:			1.0 in / s

Maximum Acceleration:		20.0 in / s²

Home Location:			0

Table Travel:				0 to 8 = X, 0 to 5.0 = Y, Z = -4.0 to 2.0

Home switch location:		0 for all Axis (Not used yet)

Home search velocity:			0.05 for all

Home latch direction:			Same used for all axis

Time to accelerate to max speed:	0.0500 s

Distance to accelerate to max speed: 	0.0250 in

Pulse rate at max speed:		64000 hz

Axis scale:				64000.0 Steps / in

200 x 16 x (1.0 ÷1.0) x 20.000 =

Test this axis

Velocity:		.07 in /s

Acceleration:		20.0 in /s²

Left and Right jog works Excellent.

Test area also works as it should.

I’m using the Chinese Blue 5-axis controller from Sain Smart. And the MD430-V1.4 stepper driver boards with the following settings. Which work awesome when running the “Test this Axis”.

SW 1 = 1, SW 2 = 1, SW 3 = 0 for current of 2.0(A)

S1 = 0, S2 = 1 Decay mode of 50⁒

S3 = 0, S4 = 1 Excitation mode of 16 microsteps

S5 = 1, S6 = 0 Torque settings of 75⁒

With all this done in Stepconf. And with the mill responding during the “Test this Axis” for each axis. When I run the Launcher, start the machine and run the LinuxCNC default G-Code. The Axis screen shows it following the G-Code as it should. But, now NONE of the steppers on the mill are moving at all.

Thank you,

Jack “I’m stuck on this one” Burnsworth