BOXFORD 190VMCxi

GENERAL SPECIFICATION

A Bench top CNC, training, 3 axis Vertical Machining Centre. The machine should be capable of cutting steel, brass, aluminium, plastic and wood with appropriate attachments. The machine should have a integral touch sensitive control panel incorporating illuminated push buttons allowing the machine to be operated without a computer being connected . An integral  safety emergency stop button is essential and a 6-station toolholder rack should be included as standard. The main drive should be a DC motor with thyristor controlled drive and overload cut-out. The axes motors should be DC stepping type controlled by bi-polar chopper drives  allowing simultaneous movement of all axis.

The machine should have full perimeter guarding with safe switches ,in the positive (safety) mode, on all access doors and guards which stop spindle and axes movements on opening.

The machine should be capable , with optional equipment, of integration with a Lathe into a Flexible Manufacturing (FMS) or a Computer Integrated Manufacturing system (CIM).

The software should include 3D solid graphics with rotation and zooming. Programming should be both through a CAD CAM package and by ISO G & M codes.

Essential Software Features

- Computer Aided Design package with Import/Export to/from most  CAD packages

- Computer Aided Manufacturing Processing package

- Manual Data Input facility

- G & M Program Import facilities

- Graphical Views and Simulations facilities, 2D, 3D, Zooming etc.

- Manufacture facility

- An optional  Off-Line Panel Emulation on screen of  Fanuc OM Control Operators Panels or Sinumerik 810/820/850/880 or Heidenhain 135/450.

- Administrator (Tutor) Functions

Specification - Mechanical

Axis travel

- X (longitudinal)		225 mm

- Y (cross)		150m

- Z (vertical)		140 mm

Spindle to column				130 mm

Distance spindle to table				202 mm

Table to column					5 mm to 130 mm

Working Table area				370 mm x 130 mm

Tee slots 					2 x 10 mm x 100 mm centres

Rapid traverse at 100%				2000mm/min

Programmable feed rate				10 mm /min to 2000 mm /min

Spindle speed range				350rpm to 3500 rpm (programmable & variable)

Tapping						0 to M4

System resolution					0.005mm

Machine resolution				0.01mm

Bed

Hardened and precision ground

Airmist Coolant System				Programmable , coolant capacity 3.37 Lt water

soluble fluid transported by local air supply .

Automatic Slideway Lubrication (optional)	Programmable timer .

## Tooling systems (optional)

Automatic toochanger 				Programmable  6 position

Tool to Tool  time				8 seconds

#### Toolholders & Tooling System

Easychange system with the following toolholders:-

6mm dia   ECV-EM 06

10mm dia   ECV-EM 10

16mm dia   ECV-EM 16

(All complete with pull studs)

## Dimensions and Weights

Machine Weight					130kg

Machine Dimensions				800 mm (L) x 680 mm (W) x 650 mm (H)

##### Specification - Electrical

Main Motor					0.45 kw ( 0.6 HP)

Spindle Motor					DC permanent magnet motor controlled by

thyristor drive package with overload protection.

Axes Motors					DC stepping type - 400 steps/rev with bi-polar

chopper drives  allowing simultaneous movement

of all axes.

Control

Personal Computer ( Not included ), the minimum specification is for Pentium P166 computer running MS-Windows’95 with 16MB RAM and 10MB space hard disc capacity, VGA colour screen, mouse, 3.5” disc drive , one free serial port and CD-ROM drive.

Standard Equipment

- Installation and Programming Manuals on CD-ROM for self-printing.

- CAD/CAM software

- Interlinking cable (machine to computer)

- Mains circuit breaker

- Machine lighting

- 6 station toolholder rack

- Tooltray

- Swarf brush, spanners and keys

- Inspection Certificate

- Earth leakage circuit breaker mains plug

Safety

- Fully enveloping transparent High Impact chip/work guards with Fail-safe switch interlocks.

- Earth leakage current breakers fitted.

- Emergency stop latching button.

- Isolator switch lockable with padlock.

- Integral electrical panel with no volt protection

- Positive end stops on all axes.

- Low voltage control circuitry.

- Ergonomic positioning of machine controls.

- Full graphical on screen test simulation prior to machining operation

### Optional Equipment

- Cabinet base with lockable door and computer shelf

- Emergency footswitch for cabinet base

- Emergency footswitch for bench model

- Workstation bench with braked castors 1500 mm (L) x 650 mm (W) x 755 mm (H)

- Airmist coolant system

- Automatic slideway lubrication

- Rotary dividing table – 4th Axis, 150 mm

- Automatic programmable 6 position toolchanger

- Automatic air powered guard

- Robot connection port

- Pneumatic vice

- Set of Toolholders comprising:

3      6mm  dia		EC-EM 06

1      10mm dia		EC-EM 10

1      16mm dia		EC-EM 16

(complete with pull studs)

- Set of Tooling comprising:-

2	Slot drill end mill       2 mm

1	Slot drill end mill       3 mm

1	Slot drill end mill       4 mm

1	Slot drill end mill       5 mm

1	Slot drill end mill       6 mm

1	Slot drill end mill       8 mm

1	Ball end mill	         6mm

2           Ball end cutters          2 mm

1	25 mm facing cutter with 16mm shank

2	Inserts for above

- Site Licence, off-line programming CAD/CAM software for 10 - 20 workstations.

## Curriculum (Optional)

The optional curriculum with standard Software package should allow:-

- In-depth and practical experience of Information Technology and Computer Control to Students

- A facility for quickly producing complex, high quality artefacts too satisfy Design and Make tasks

- An opportunity to manufacture artefacts for practical use in Fabrics and Food Technology.

- A Platform for creating links with Art, Science, Business Studies, Electronic and Industry.

- A practical, real life application of mathematics

And offer:-

- 10 Session Curriculum  in 3 levels taking the Student from novice to expert user via a systematic and easy to follow route. All curriculum should be fully supported by detailed Teacher Notes and Students Workbooks on CD-ROM for self printing .

- Extensive library of fully prepared components complete with program disk and illustrated step by step guide of the manufacture and assembly procedure.

- Extensive library of detailed project material including Teacher resources, Lessons structures and Student worksheets.

With optional Student Projects which should include:

- Crocodile

- Desk tidy

- Seal

- Shark

- Floppy disc rack

- Compact disc rack

- Maze game

- Duck

- Letter/ toast rack

CAD/CAM Software

Windows 3.0 or above compatibility, the software should be an integrated suite of powerful CAD/CAM tools which create G&M code programs conforming to Industrial ISO programming standards and should include the following features:

Computer Aided Design

	The CAD package should  provide a number of tools which allows the input of :- Straight Lines, Arcs, Circles, Bezier Curves, Various Polygons and Text (any available True Type font)

	Manipulation tools allow elements to be :- Moved, Copied, Mirrored, Rotated, Distorted, Trimmed and replicated in both Rectangular and Circular Arrays (e.g. PCD Holes).

	A Smart Fill tool is provided to fill pocketed areas with colour, taking into account any islands that may exist.

	Alternatively, component profiles can be imported from a third party CAD package (e.g. Autocad) using DXF, WMF, EMF, HPGL or AFF file formats.

CAM Processing

	Pocketed areas (which may include multi-islands) to be removed or profiled, are assigned  a depth and machining strategy which includes Remove Area, Profile Inside only, Profile Outside only or Peck Drill.

	The CAM Processor automatically produces an Error free G&M code listing making optimum use of the cutting tools available and assigning feeds, speeds and cut depths appropriate to the selected material and the machine tool been used.

	Total Cut Depths assigned to each individual Cutting Tool will NEVER exceed the flute length of the cutter.

Manual Data Input

	Existing G&M Code programs can be edited, or new ones defined using the sophisticated CNC editor.

	Blocks can be modified, inserted or deleted with the aid of the on-line programming code help facility.

	When Inputting new programs, each block entered produces a graphical simulation of the cutter path.

	Automatic error checking ensures incomplete blocks and speeds, feeds and cut depths inappropriate to the material and machine tool cannot be specified.

G&M Program Import

2½D or 3D Post Processed files can be imported from major CAD/CAM packages.  The  machines should include integrated Continuous Path Manufacture feature ensures the programs will be manufactured very efficiently (see manufacture section).

Graphical Views and Simulations

The component produced by a program can be viewed in both 2D and 3D.  A Zoom facility and cross-sectional views allow individual sections of the component to be viewed in more detail.

The manufacture cycle for programs can be simulated in 2D and 3D in either single block

mode or continuous mode.

Manufacture

PC machines incorporate Datum Switches which allow the machine to automatically initialise at the start of manufacture.  The only user input is the total measured height of the workpiece and clamping device.  In this way, the machining cycle is initiated effortlessly and quickly.

PC machines incorporate hardware capable of continuous 3 axis movement and utilise Continuous Path Manufacture allowing large programs imported from Major CAD/CAM packages to be machined extremely efficiently.  This is particularly useful for Rapid Prototyping of 3D reliefs.

The machines should include incorporate an ergonomically positioned operators panel which controls the machine tool functions.  This should emulates the type of manual control panel found on industrial CNC controls.

Off-Line Fanuc Panel Emulation

This software package can be integrated into the CAD/CAM software suite to allow programs to be edited or written using an on-screen emulation of the Fanuc OM Control Operators Panel.

Manual control of the machine tool and automatic manufacture of programs can be simulated

using the emulation software.

Administrator (Tutor) Functions

In administration mode, the following can be modified.

TOOL Library - Defining of new tools and offset setting.

CUT Data - This is the information the sophisticated CAM Processor uses to automatically create a G&M code program.  Administrators can Add/Delete materials, change feeds, speeds and cut depths.

PARK Position - Tool Position for Tool Changes

UNITS - Sets the units for the Configuration settings - metric or imperial.