[EMC]

MACHINE = GSF1000-5

DEBUG = 0

VERSION = 1.1

[DISPLAY]

DISPLAY = gmoccapy

POSITION_OFFSET = RELATIVE

POSITION_FEEDBACK = ACTUAL

MAX_FEED_OVERRIDE = 2.000000

MAX_SPINDLE_OVERRIDE = 1.000000

MIN_SPINDLE_OVERRIDE = 0.500000

INTRO_GRAPHIC = linuxcnc.gif

INTRO_TIME = 5

PROGRAM_PREFIX = /home/mill/linuxcnc/nc_files

INCREMENTS = .1in .01in 1in 6in 8in 12in

POSITION_FEEDBACK = ACTUAL

DEFAULT_LINEAR_VELOCITY = 0.250000

MAX_LINEAR_VELOCITY = 1.000000

MIN_LINEAR_VELOCITY = 0.016670

DEFAULT_ANGULAR_VELOCITY = 12.000000

MAX_ANGULAR_VELOCITY = 180.000000

MIN_ANGULAR_VELOCITY = 1.666667

EDITOR = gedit

GEOMETRY = xyz

CYCLE_TIME = 100

[FILTER]

PROGRAM_EXTENSION = .png,.gif,.jpg Greyscale Depth Image

PROGRAM_EXTENSION = .py Python Script

png = image-to-gcode

gif = image-to-gcode

jpg = image-to-gcode

py = python

[TASK]

TASK = milltask

CYCLE_TIME = 0.010

[RS274NGC]

PARAMETER_FILE = linuxcnc.var

RS274NGC_STARTUP_CODE = G20 G40 G90 G94 G97 G64 P0.001

[EMCMOT]

EMCMOT = motmod

COMM_TIMEOUT = 1.0

SERVO_PERIOD = 1000000

[HMOT]

CARD0=hm2_7i96s.0

[HAL]

HALUI = halui

HALFILE = GSF1000-5.hal

HALFILE = custom.hal

POSTGUI_HALFILE = gmoccapy_postgui.hal

POSTGUI_HALFILE = custom_postgui.hal

SHUTDOWN = shutdown.hal

[HALUI]

[KINS]

JOINTS = 3

KINEMATICS = trivkins coordinates=XYZ

[TRAJ]

COORDINATES =  XYZ

LINEAR_UNITS = inch

ANGULAR_UNITS = degree

DEFAULT_LINEAR_VELOCITY = 1.67

MAX_LINEAR_VELOCITY = 16.67

NO_FORCE_HOMING = 1

[EMCIO]

EMCIO = io

CYCLE_TIME = 0.100

TOOL_TABLE = tool.tbl

#******************************************

[AXIS_X]

MAX_VELOCITY = 16.666666666666668

MAX_ACCELERATION = 60.0

MIN_LIMIT = -0.0

MAX_LIMIT = 24.0

[JOINT_0]

TYPE = LINEAR

HOME = 12.0

FERROR = 0.5

MIN_FERROR = 0.05

MAX_VELOCITY = 16.666666666666668

MAX_ACCELERATION = 60.0

# The values below should be 25% larger than MAX_VELOCITY and MAX_ACCELERATION

# If using BACKLASH compensation STEPGEN_MAXACCEL should be 100% larger.

STEPGEN_MAXVEL = 33.33

STEPGEN_MAXACCEL = 120.00

P = 1000.0

I = 0.0

D = 0.0

FF0 = 0.0

FF1 = 1.0

FF2 = 0.0

BIAS = 0.0

DEADBAND = 0.0

MAX_OUTPUT = 0.0

# these are in nanoseconds

DIRSETUP   = 5000

DIRHOLD    = 5000

STEPLEN    = 2000

STEPSPACE  = 8000

STEP_SCALE = -1310.9677

BACKLASH = 0.0

MIN_LIMIT = -0.0

MAX_LIMIT = 24.0

HOME_OFFSET = 1.000000

HOME_SEARCH_VEL = -30

HOME_LATCH_VEL = -0.016667

HOME_FINAL_VEL = 10.000000

HOME_USE_INDEX = YES

HOME_IGNORE_LIMITS = YES

HOME_SEQUENCE = 2

#******************************************

#******************************************

[AXIS_Y]

MAX_VELOCITY = 3.3333333333333335

MAX_ACCELERATION = 45.0

MIN_LIMIT = -0.0

MAX_LIMIT = 11.0

[JOINT_1]

TYPE = LINEAR

HOME = 6.0

FERROR = 0.5

MIN_FERROR = 0.05

MAX_VELOCITY = 3.3333333333333335

MAX_ACCELERATION = 45.0

# The values below should be 25% larger than MAX_VELOCITY and MAX_ACCELERATION

# If using BACKLASH compensation STEPGEN_MAXACCEL should be 100% larger.

STEPGEN_MAXVEL = 6.67

STEPGEN_MAXACCEL = 90.00

P = 1000.0

I = 0.0

D = 0.0

FF0 = 0.0

FF1 = 1.0

FF2 = 0.0

BIAS = 0.0

DEADBAND = 0.0

MAX_OUTPUT = 0.0

# these are in nanoseconds

DIRSETUP   = 5000

DIRHOLD    = 5000

STEPLEN    = 2000

STEPSPACE  = 8000

STEP_SCALE = 4063.9999

BACKLASH = 0.0

MIN_LIMIT = -0.0

MAX_LIMIT = 11.0

HOME_OFFSET = 1.000000

HOME_SEARCH_VEL = -30

HOME_LATCH_VEL = -0.016667

HOME_FINAL_VEL = 10.000000

HOME_USE_INDEX = YES

HOME_IGNORE_LIMITS = YES

HOME_SEQUENCE = 3

#******************************************

#******************************************

[AXIS_Z]

MAX_VELOCITY = 5.666666666666667

MAX_ACCELERATION = 100.0

MIN_LIMIT = -4.0

MAX_LIMIT = 7.0

[JOINT_2]

TYPE = LINEAR

HOME = 0.0

FERROR = 0.5

MIN_FERROR = 0.05

MAX_VELOCITY = 5.666666666666667

MAX_ACCELERATION = 100.0

# The values below should be 25% larger than MAX_VELOCITY and MAX_ACCELERATION

# If using BACKLASH compensation STEPGEN_MAXACCEL should be 100% larger.

STEPGEN_MAXVEL = 11.33

STEPGEN_MAXACCEL = 200.00

P = 1000.0

I = 0.0

D = 0.0

FF0 = 0.0

FF1 = 1.0

FF2 = 0.0

BIAS = 0.0

DEADBAND = 0.0

MAX_OUTPUT = 0.0

# these are in nanoseconds

DIRSETUP   = 5000

DIRHOLD    = 5000

STEPLEN    = 2000

STEPSPACE  = 8000

STEP_SCALE = 4031.7459

BACKLASH = 0.0

MIN_LIMIT = -4.0

MAX_LIMIT = 7.0

HOME_OFFSET = 1.000000

HOME_SEARCH_VEL = -30

HOME_LATCH_VEL = -0.016667

HOME_FINAL_VEL = 10.000000

HOME_USE_INDEX = YES

HOME_IGNORE_LIMITS = YES

HOME_SEQUENCE = 1

#******************************************

[SPINDLE_0]

MAX_VELOCITY = 100.0

MAX_ACCELERATION = 300.0

# The values below should be 25% larger than MAX_VELOCITY and MAX_ACCELERATION

# If using BACKLASH compensation STEPGEN_MAXACCEL should be 100% larger.

STEPGEN_MAXVEL = 125.00

STEPGEN_MAXACCEL = 375.00

P = 0.0

I = 0.0

D = 0.0

FF0 = 1.0

FF1 = 0.0

FF2 = 0.0

BIAS = 0.0

DEADBAND = 0.0

MAX_OUTPUT = 625.0

# these are in nanoseconds

DIRSETUP   = 10000

DIRHOLD    = 10000

STEPLEN    = 5000

STEPSPACE  = 5000

STEP_SCALE = 1000.0