loadrt [KINS]KINEMATICS

loadrt [EMCMOT]EMCMOT servo_period_nsec=[EMCMOT]SERVO_PERIOD num_joints=[KINS]J>

loadrt hostmot2

loadrt hm2_eth board_ip="10.10.10.10" config="num_encoders=1 num_pwmgens=1 num_>

setp    [HMOT](CARD0).pwmgen.pwm_frequency 20000

setp    [HMOT](CARD0).pwmgen.pdm_frequency 6000000

setp    [HMOT](CARD0).watchdog.timeout_ns 5000000

loadusr -W hal_input -KRAL Microsoft

loadrt pid names=pid.x,pid.y,pid.z,pid.s

loadrt abs names=abs.spindle

loadrt lowpass names=lowpass.spindle

loadrt classicladder_rt numPhysInputs=15 numPhysOutputs=15 numS32in=10 numS32ou>

loadrt mux16 names=jogspeed

addf [HMOT](CARD0).read          servo-thread

addf motion-command-handler   servo-thread

addf motion-controller        servo-thread

addf pid.x.do-pid-calcs       servo-thread

addf pid.y.do-pid-calcs       servo-thread

addf pid.z.do-pid-calcs       servo-thread

addf pid.s.do-pid-calcs       servo-thread

addf classicladder.0.refresh servo-thread

addf jogspeed                 servo-thread

addf abs.spindle              servo-thread

addf lowpass.spindle          servo-thread

addf [HMOT](CARD0).write         servo-thread

setp [HMOT](CARD0).dpll.01.timer-us -50

setp [HMOT](CARD0).stepgen.timer-number 1

# external output signals

# --- SPINDLE-ENABLE ---

net spindle-enable  =>     [HMOT](CARD0).outm.00.out-05

# external input signals

# --- MIN-Z ---

net min-z     <=  [HMOT](CARD0).gpio.000.in_not

# --- MAX-HOME-Z ---

net max-home-z     <=  [HMOT](CARD0).gpio.001.in_not

# --- MIN-X ---

net min-x     <=  [HMOT](CARD0).gpio.002.in_not

# --- MAX-HOME-X ---

net max-home-x     <=  [HMOT](CARD0).gpio.003.in_not

# --- MIN-Y ---

# --- MIN-Y ---

net min-y     <=  [HMOT](CARD0).gpio.004.in_not

# --- MAX-HOME-Y ---

net max-home-y     <=  [HMOT](CARD0).gpio.005.in_not

# --- ESTOP-EXT ---

net estop-ext     <=  [HMOT](CARD0).gpio.006.in

#*******************

#  AXIS X JOINT 0

#*******************

setp   pid.x.Pgain     [JOINT_0]P

setp   pid.x.Igain     [JOINT_0]I

setp   pid.x.Dgain     [JOINT_0]D

setp   pid.x.bias      [JOINT_0]BIAS

setp   pid.x.FF0       [JOINT_0]FF0

setp   pid.x.FF1       [JOINT_0]FF1

setp   pid.x.FF2       [JOINT_0]FF2

setp   pid.x.deadband  [JOINT_0]DEADBAND

setp   pid.x.maxoutput [JOINT_0]MAX_OUTPUT

setp   pid.x.error-previous-target true

# This setting is to limit bogus stepgen

# velocity corrections caused by position

# feedback sample time jitter.

setp   pid.x.maxerror 0.000500

net x-index-enable  =>  pid.x.index-enable

net x-enable        =>  pid.x.enable

net x-pos-cmd       =>  pid.x.command

net x-pos-fb        =>  pid.x.feedback

net x-output        <=  pid.x.output

# Step Gen signals/setup

setp   [HMOT](CARD0).stepgen.01.dirsetup        [JOINT_0]DIRSETUP

setp   [HMOT](CARD0).stepgen.01.dirhold         [JOINT_0]DIRHOLD

setp   [HMOT](CARD0).stepgen.01.steplen         [JOINT_0]STEPLEN

setp   [HMOT](CARD0).stepgen.01.stepspace       [JOINT_0]STEPSPACE

setp   [HMOT](CARD0).stepgen.01.position-scale  [JOINT_0]STEP_SCALE

setp   [HMOT](CARD0).stepgen.01.step_type        0

setp   [HMOT](CARD0).stepgen.01.control-type     1

setp   [HMOT](CARD0).stepgen.01.maxaccel         [JOINT_0]STEPGEN_MAXACCEL

setp   [HMOT](CARD0).stepgen.01.maxvel           [JOINT_0]STEPGEN_MAXVEL

# ---closedloop stepper signals---

net x-pos-cmd    <= joint.0.motor-pos-cmd

net x-vel-cmd    <= joint.0.vel-cmd

net x-output     => [HMOT](CARD0).stepgen.01.velocity-cmd

net x-pos-fb     <= [HMOT](CARD0).stepgen.01.position-fb

net x-pos-fb     => joint.0.motor-pos-fb

net x-enable     <= joint.0.amp-enable-out

net x-enable     => [HMOT](CARD0).stepgen.01.enable

# ---setup home / limit switch signals---

net max-home-x     =>  joint.0.home-sw-in

net min-x     =>  joint.0.neg-lim-sw-in

net max-home-x     =>  joint.0.pos-lim-sw-in

#*******************

#  AXIS Y JOINT 1

#*******************

setp   pid.y.Pgain     [JOINT_1]P

setp   pid.y.Igain     [JOINT_1]I

setp   pid.y.Dgain     [JOINT_1]D

setp   pid.y.bias      [JOINT_1]BIAS

setp   pid.y.FF0       [JOINT_1]FF0

setp   pid.y.FF1       [JOINT_1]FF1

setp   pid.y.FF2       [JOINT_1]FF2

setp   pid.y.deadband  [JOINT_1]DEADBAND

setp   pid.y.maxoutput [JOINT_1]MAX_OUTPUT

setp   pid.y.error-previous-target true

# This setting is to limit bogus stepgen

# velocity corrections caused by position

# feedback sample time jitter.

setp   pid.y.maxerror 0.000500

net y-index-enable  =>  pid.y.index-enable

net y-enable        =>  pid.y.enable

net y-pos-cmd       =>  pid.y.command

net y-pos-fb        =>  pid.y.feedback

net y-output        <=  pid.y.output

# Step Gen signals/setup

setp   [HMOT](CARD0).stepgen.00.dirsetup        [JOINT_1]DIRSETUP

setp   [HMOT](CARD0).stepgen.00.dirhold         [JOINT_1]DIRHOLD

setp   [HMOT](CARD0).stepgen.00.steplen         [JOINT_1]STEPLEN

setp   [HMOT](CARD0).stepgen.00.stepspace       [JOINT_1]STEPSPACE

setp   [HMOT](CARD0).stepgen.00.position-scale  [JOINT_1]STEP_SCALE

setp   [HMOT](CARD0).stepgen.00.step_type        0

setp   [HMOT](CARD0).stepgen.00.control-type     1

setp   [HMOT](CARD0).stepgen.00.maxaccel         [JOINT_1]STEPGEN_MAXACCEL

setp   [HMOT](CARD0).stepgen.00.maxvel           [JOINT_1]STEPGEN_MAXVEL

# ---closedloop stepper signals---

net y-pos-cmd    <= joint.1.motor-pos-cmd

net y-vel-cmd    <= joint.1.vel-cmd

net y-output     => [HMOT](CARD0).stepgen.00.velocity-cmd

net y-pos-fb     <= [HMOT](CARD0).stepgen.00.position-fb

net y-pos-fb     => joint.1.motor-pos-fb

net y-enable     <= joint.1.amp-enable-out

net y-enable     => [HMOT](CARD0).stepgen.00.enable

# ---setup home / limit switch signals---

net max-home-y     =>  joint.1.home-sw-in

net min-y     =>  joint.1.neg-lim-sw-in

net max-home-y     =>  joint.1.pos-lim-sw-in

#*******************

#  AXIS Z JOINT 2

#*******************

setp   pid.z.Pgain     [JOINT_2]P

setp   pid.z.Igain     [JOINT_2]I

setp   pid.z.Dgain     [JOINT_2]D

setp   pid.z.bias      [JOINT_2]BIAS

setp   pid.z.FF0       [JOINT_2]FF0

setp   pid.z.FF1       [JOINT_2]FF1

setp   pid.z.FF2       [JOINT_2]FF2

setp   pid.z.deadband  [JOINT_2]DEADBAND

setp   pid.z.maxoutput [JOINT_2]MAX_OUTPUT

setp   pid.z.error-previous-target true

# This setting is to limit bogus stepgen

# velocity corrections caused by position

# feedback sample time jitter.

setp   pid.z.maxerror 0.000500

net z-index-enable  =>  pid.z.index-enable

net z-enable        =>  pid.z.enable

net z-pos-cmd       =>  pid.z.command

net z-pos-fb        =>  pid.z.feedback

net z-output        <=  pid.z.output

# Step Gen signals/setup

setp   [HMOT](CARD0).stepgen.02.dirsetup        [JOINT_2]DIRSETUP

setp   [HMOT](CARD0).stepgen.02.dirhold         [JOINT_2]DIRHOLD

setp   [HMOT](CARD0).stepgen.02.steplen         [JOINT_2]STEPLEN

setp   [HMOT](CARD0).stepgen.02.stepspace       [JOINT_2]STEPSPACE

setp   [HMOT](CARD0).stepgen.02.position-scale  [JOINT_2]STEP_SCALE

setp   [HMOT](CARD0).stepgen.02.step_type        0

setp   [HMOT](CARD0).stepgen.02.control-type     1

setp   [HMOT](CARD0).stepgen.02.maxaccel         [JOINT_2]STEPGEN_MAXACCEL

setp   [HMOT](CARD0).stepgen.02.maxvel           [JOINT_2]STEPGEN_MAXVEL

# ---closedloop stepper signals---

net z-pos-cmd    <= joint.2.motor-pos-cmd

net z-vel-cmd    <= joint.2.vel-cmd

net z-output     => [HMOT](CARD0).stepgen.02.velocity-cmd

net z-pos-fb     <= [HMOT](CARD0).stepgen.02.position-fb

net z-pos-fb     => joint.2.motor-pos-fb

net z-enable     <= joint.2.amp-enable-out

net z-enable     => [HMOT](CARD0).stepgen.02.enable

# ---setup home / limit switch signals---

net max-home-z     =>  joint.2.home-sw-in

net min-z     =>  joint.2.neg-lim-sw-in

net max-home-z     =>  joint.2.pos-lim-sw-in

#*******************

#  SPINDLE

#*******************

setp   pid.s.Pgain     [SPINDLE_0]P

setp   pid.s.Igain     [SPINDLE_0]I

setp   pid.s.Dgain     [SPINDLE_0]D

setp   pid.s.bias      [SPINDLE_0]BIAS

setp   pid.s.FF0       [SPINDLE_0]FF0

setp   pid.s.FF1       [SPINDLE_0]FF1

setp   pid.s.FF2       [SPINDLE_0]FF2

setp   pid.s.deadband  [SPINDLE_0]DEADBAND

setp   pid.s.maxoutput [SPINDLE_0]MAX_OUTPUT

setp   pid.s.error-previous-target true

# This setting is to limit bogus stepgen

# velocity corrections caused by position

# feedback sample time jitter.

setp   pid.s.maxerror 0.000500

net spindle-index-enable  =>  pid.s.index-enable

net spindle-enable        =>  pid.s.enable

net spindle-vel-cmd-rpm     => pid.s.command

net spindle-vel-fb-rpm      => pid.s.feedback

net spindle-output        <=  pid.s.output

# Step Gen signals/setup

setp   [HMOT](CARD0).stepgen.04.dirsetup        [SPINDLE_0]DIRSETUP

setp   [HMOT](CARD0).stepgen.04.dirhold         [SPINDLE_0]DIRHOLD

setp   [HMOT](CARD0).stepgen.04.steplen         [SPINDLE_0]STEPLEN

setp   [HMOT](CARD0).stepgen.04.stepspace       [SPINDLE_0]STEPSPACE

setp   [HMOT](CARD0).stepgen.04.position-scale  [SPINDLE_0]STEP_SCALE

setp   [HMOT](CARD0).stepgen.04.step_type        0

setp   [HMOT](CARD0).stepgen.04.control-type     1

setp   [HMOT](CARD0).stepgen.04.maxaccel         [SPINDLE_0]STEPGEN_MAXACCEL

setp   [HMOT](CARD0).stepgen.04.maxvel           [SPINDLE_0]STEPGEN_MAXVEL

setp   [HMOT](CARD0).stepgen.04.step.invert_output   true

net machine-is-on           =>  [HMOT](CARD0).stepgen.04.enable

net spindle-vel-cmd-rps     =>  [HMOT](CARD0).stepgen.04.velocity-cmd

net spindle-vel-fb-rps         <=  [HMOT](CARD0).stepgen.04.velocity-fb

# ---setup spindle control signals---

net spindle-vel-cmd-rps        <=  spindle.0.speed-out-rps

net spindle-vel-cmd-rps-abs    <=  spindle.0.speed-out-rps-abs

net spindle-vel-cmd-rpm        <=  spindle.0.speed-out

net spindle-vel-cmd-rpm-abs    <=  spindle.0.speed-out-abs

net spindle-enable             <=  spindle.0.on

net spindle-cw                 <=  spindle.0.forward

net spindle-ccw                <=  spindle.0.reverse

net spindle-brake              <=  spindle.0.brake

net spindle-revs               =>  spindle.0.revs

net spindle-at-speed           =>  spindle.0.at-speed

net spindle-vel-fb-rps         =>  spindle.0.speed-in

net spindle-index-enable      <=>  spindle.0.index-enable

# ---Setup spindle at speed signals---

sets spindle-at-speed true

#******************************

# connect miscellaneous signals

#******************************

#  ---HALUI signals---

net axis-select-x  halui.axis.x.select

net jog-x-pos      halui.axis.x.plus

net jog-x-neg      halui.axis.x.minus

net jog-x-analog   halui.axis.x.analog

net x-is-homed     halui.joint.0.is-homed

net axis-select-y  halui.axis.y.select

net jog-y-pos      halui.axis.y.plus

net jog-y-neg      halui.axis.y.minus

net jog-y-analog   halui.axis.y.analog

net y-is-homed     halui.joint.1.is-homed

net axis-select-z  halui.axis.z.select

net jog-z-pos      halui.axis.z.plus

net jog-z-neg      halui.axis.z.minus

net jog-z-analog   halui.axis.z.analog

net z-is-homed     halui.joint.2.is-homed

net jog-selected-pos      halui.axis.selected.plus

net jog-selected-neg      halui.axis.selected.minus

net spindle-manual-cw     halui.spindle.0.forward

net spindle-manual-ccw    halui.spindle.0.reverse

net spindle-manual-stop   halui.spindle.0.stop

net machine-is-on         halui.machine.is-on

net jog-speed             halui.axis.jog-speed

net MDI-mode              halui.mode.is-mdi

#  ---coolant signals---

net coolant-mist      <=  iocontrol.0.coolant-mist

net coolant-flood     <=  iocontrol.0.coolant-flood

#  ---probe signal---

net probe-in     =>  motion.probe-input

# ---USB device jog button signals---

# connect selectable mpg jog speeds

net jog-speed-a           =>  jogspeed.sel0

net jog-speed-b           =>  jogspeed.sel1

net jog-speed             <=  jogspeed.out-f

setp    jogspeed.in00          0.100000

setp    jogspeed.in01          1.000000

setp    jogspeed.in02          10.000000

setp    jogspeed.in03          100.000000

net jog-x-pos            input.0.btn-b

net jog-x-neg            input.0.btn-x

net jog-y-pos            input.0.btn-y

net jog-y-neg            input.0.btn-a

net jog-z-pos            input.0.btn-tl

net jog-z-neg            input.0.btn-tr

#  ---motion control signals---

net in-position               <=  motion.in-position

net machine-is-enabled        <=  motion.motion-enabled

#  ---digital in / out signals---

#  ---estop signals---

net estop-out     <=  iocontrol.0.user-enable-out

net estop-ext     =>  iocontrol.0.emc-enable-in

#  ---manual tool change signals---

net tool-change-request    <= iocontrol.0.tool-change

net tool-change-confirmed  => iocontrol.0.tool-changed

net tool-number            <= iocontrol.0.tool-prep-number

#  ---ignore tool prepare requests---

net tool-prepare-loopback   iocontrol.0.tool-prepare      =>  iocontrol.0.tool->

# Load Classicladder without GUI (can reload LADDER GUI in AXIS GUI

loadusr classicladder --nogui custom.clp